Hazard Analysis and Risk Assessment (HARA)

Electric Park Shuttle Vehicle

Document ID: HARA-002
Vehicle: Small electric bus (park shuttle)
Date: October 30, 2025
ISO 26262 Reference: Part 3 - Concept Phase
Supersedes: HARA-001

1. Vehicle Description

Vehicle Type: Small open-sided shuttle bus operating in controlled park environment
Maximum Speed: 25 mph (40 km/h)
Passenger Capacity: 20 persons
Operating Environment: Closed park (controlled environment, no public roads)
Annual Mileage: 3,000 miles
Driver Profile: Professional trained operators with high EV system awareness

Special Characteristics:

- Open-sided design (easy emergency egress)

- Retrofit from gasoline to electric

- Low speed operation only

- Controlled, predictable routes

- Frequent steep hills/grades

Key Safety Features:

- Redundant braking system (service + parking brake)

- Manual steering backup (hydraulic/mechanical)

- Can coast safely to stop

2. HARA Methodology

2.1 Severity Classification (S)

- S0: No injuries

- S1: Light and moderate injuries

- S2: Severe and life-threatening injuries (survival probable)

- S3: Life-threatening injuries (survival uncertain), fatal injuries

2.2 Exposure Classification (E)

- E0: Incredible (less than once per lifetime of vehicle)

- E1: Very low probability (once per vehicle lifetime)

- E2: Low probability (once per year)

- E3: Medium probability (once per month)

- E4: High probability (once per week or more)

2.3 Controllability Classification (C)

- C0: Controllable in general

- C1: Simply controllable (>99% of drivers can avoid harm)

- C2: Normally controllable (>90% of drivers can avoid harm)

- C3: Difficult to control or uncontrollable (<90% can avoid harm)

2.4 ASIL Classification Matrix (ISO 26262-3 Table D.1)

The following matrix shows how Severity, Exposure, and Controllability combine to determine ASIL:

| Severity | Exposure | C0 | C1 | C2 | C3 |
| --- | --- | --- | --- | --- | --- |
| S1 | E1 | QM | QM | QM | A |
| S1 | E2 | QM | QM | A | B |
| S1 | E3 | QM | A | B | C |
| S1 | E4 | QM | B | C | C |
| S2 | E1 | QM | QM | A | B |
| S2 | E2 | QM | A | B | C |
| S2 | E3 | A | B | C | D |
| S2 | E4 | B | C | D | D |
| S3 | E1 | QM | A | B | D |
| S3 | E2 | A | B | C | D |
| S3 | E3 | B | C | D | D |
| S3 | E4 | C | D | D | D |

Note: S0 (no injuries) always results in QM (Quality Management) regardless of E and C.

3. Hazardous Events Analysis

3.1 Propulsion System Hazards

H-01: Unintended Acceleration

Hazardous Event: Vehicle accelerates without driver command or fails to respond to brake application

Operational Situation: Any driving situation, particularly when stopping for passengers or navigating crowded areas

Severity (S2):

- Low speed (25 mph max) limits kinetic energy

- Open-sided vehicle allows passengers to potentially jump clear

- However, passengers at risk, pedestrians in park environment

- Likely severe injuries if collision occurs

Exposure (E3):

- Vehicle operates frequently (though low annual mileage)

- Fault could occur at any time during operation

- Medium probability: ~once per month of operation

Controllability (C2):

- Professional trained driver can apply brakes

- Redundant braking system available

- Low speed provides more reaction time

- Emergency stop procedures known to driver

- Normally controllable (>90% of trained drivers can mitigate)

ASIL: C (per ISO 26262-3 Table D.1: S2 + E3 + C2)

FTTI: 100ms

Safety Goal SG-01: The vehicle shall prevent unintended acceleration and ensure driver commands have priority over all other control inputs.

H-02: Loss of Propulsion on Hill

Hazardous Event: Drive system disables while vehicle is on steep grade, potentially causing rollback

Operational Situation: Climbing or descending steep hills (frequent in park operation)

Severity (S1):

- Low speed and mass limits rollback speed

- Parking brake and service brake available as backup

- Open-sided design allows egress

- Likely light to moderate injuries if rollback occurs

Exposure (E4):

- Vehicle frequently operates on steep hills

- High probability: could occur weekly or more if battery/drive issues exist

Controllability (C1):

- Driver trained to apply parking brake

- Service brake still functional (independent of propulsion)

- Low speed and trained operator = simply controllable

- 99% of trained drivers can control

ASIL: B (per ISO 26262-3 Table D.1: S1 + E4 + C1)

FTTI: 200ms

Safety Goal SG-02: The vehicle shall provide sufficient warning before loss of propulsion to allow driver to safely stop, and shall not disable propulsion without adequate notice except in immediate danger situations.

H-03: Loss of Propulsion in Normal Driving

Hazardous Event: Drive system disables during normal cruise/acceleration on flat terrain

Operational Situation: Normal park operations on level ground

Severity (S0):

- Low speed allows safe coasting to stop

- No traffic following (controlled environment)

- No significant harm expected in park environment

- Inconvenience only

Exposure (E3):

- Medium probability if battery/drive monitoring has issues

Controllability (C0):

- Controllable in general - vehicle coasts safely

- Driver can steer to safe location

- Service brake functional

ASIL: QM (Quality Management - no ASIL required per ISO 26262-3 Table D.1: S0)

Safety Goal: None required (QM level design practices sufficient)

Note: While no ASIL required, should still provide driver notification and graceful degradation.

H-04: Unintended Regenerative Braking (Motor Reaction)

Hazardous Event: Motor controller applies unexpected regenerative torque (negative torque) without driver command, causing sudden deceleration

Operational Situation: Any driving situation, particularly:

- Cruising at steady speed with standing passengers

- During passenger boarding/alighting (vehicle moving slowly)

- Throttle release transitions (driver lifts foot from accelerator)

Severity (S1):

- Regenerative braking is limited in magnitude compared to friction brakes

- Max regen typically 0.2-0.3g deceleration (vs 0.8g+ for friction brakes)

- At 25 mph, gentler deceleration than emergency braking

- Standing passengers may stumble but unlikely to fall completely

- Light to moderate injuries possible (bruises, minor impacts with interior)

- Not severe injuries - regen isn't strong enough

Exposure (E3):

- Software or sensor faults could trigger at any time during operation

- Motor controller receives torque commands continuously

- Medium probability: once per month if control logic has issues

- Higher exposure than friction brake fault (motor commands are continuous)

Controllability (C1):

- Driver can immediately apply throttle to counteract

- Regen torque is gradual compared to friction brakes (motor current ramps)

- Low speed (25 mph) provides more reaction time

- Professional trained driver recognizes behavior quickly

- Simply controllable (>99% of trained drivers can mitigate)

- Much easier to control than sudden friction brake application

ASIL: B (per ISO 26262-3 Table D.1: S1 + E3 + C1)

FTTI: 200ms (driver reaction + motor controller response time)

Safety Goal SG-03: The vehicle shall prevent unintended motor torque (both positive and negative) and ensure motor torque commands reflect driver intent only.

3.2 Battery System Hazards

H-05: Battery Thermal Runaway / Fire

Hazardous Event: Battery cells overheat leading to thermal runaway, fire, and toxic gas release

Operational Situation: Any operating condition, but higher risk during:

- Charging (not while passengers aboard)

- High power demand (hill climbing)

- Extreme ambient temperatures (rare per operational data)

Severity (S3):

- Fire with 20 passengers on board

- Toxic gas from LFP chemistry (less severe than NMC but still dangerous)

- Open-sided design aids egress but fire/smoke still life-threatening

- Fatal injuries possible

Exposure (E1):

- Very low probability with proper BMS

- LFP chemistry more stable than other lithium-ion chemistries

- Low annual mileage reduces exposure

- Once per vehicle lifetime probability

Controllability (C2):

- Open-sided design allows rapid egress

- Professional driver trained in emergency procedures

- Vehicle can be stopped immediately

- However, fire spreads quickly, smoke/gas dangerous

- Normally controllable with training

ASIL: D (per ISO 26262-3 Table D.1: S3 + E1 + C2)

FTTI: 50ms

Safety Goal SG-04: The vehicle shall detect battery over-temperature conditions and transition to a safe state (disable charging/discharging) before thermal runaway occurs.

H-06: Battery Over-Voltage (Cell Level)

Hazardous Event: Battery cells charged above safe voltage, leading to degradation or potential thermal event

Operational Situation: During charging or regenerative braking

Severity (S2):

- Could lead to cell damage and eventual thermal event (delayed hazard)

- Immediate risk lower than direct thermal runaway

- Severe injuries possible if leads to fire later

Exposure (E2):

- Low probability with proper BMS and charging control

- Could occur once per year if charging system has issues

Controllability (C1):

- BMS monitoring can detect and prevent

- Charging occurs when passengers not aboard (controllable)

- If during regen braking, driver can release accelerator

- Simply controllable

ASIL: C (per ISO 26262-3 Table D.1: S2 + E2 + C1)

FTTI: 100ms

Safety Goal SG-05: The vehicle shall prevent battery cell voltages from exceeding safe limits during all charging and discharging operations.

H-07: Battery Under-Voltage (Cell Level)

Hazardous Event: Battery cells discharged below safe voltage, causing permanent damage and potential safety issues

Operational Situation: Extended operation with low state of charge

Severity (S1):

- Primarily damages battery (economic impact)

- Could lead to stranded vehicle

- Low immediate safety risk in controlled park environment

- Light injuries possible if vehicle becomes inoperable in unsafe location

Exposure (E2):

- Low probability with proper BMS monitoring

- Once per year if monitoring/warnings insufficient

Controllability (C0):

- Driver receives multiple warnings before critical level

- Can navigate to charging location

- Park environment allows safe stopping anywhere

- Controllable in general

ASIL: QM (per ISO 26262-3 Table D.1: S1 + E2 + C0)

Safety Goal: None required (QM level)

Note: Should implement multi-level warnings to driver (QM requirement).

H-08: Battery Over-Temperature (Non-Fire)

Hazardous Event: Battery temperature exceeds normal operating range but below thermal runaway threshold

Operational Situation: High power demand (hill climbing) or extreme ambient temperature

Severity (S1):

- Accelerates battery aging

- May lead to reduced performance

- Not immediately dangerous but could progress to H-05

- Light injuries possible if leads to sudden loss of power on hill

Exposure (E3):

- Medium probability given frequent hill climbing

- Could occur monthly during summer operation

Controllability (C1):

- BMS can detect and limit power

- Driver can reduce demand

- System can transition to reduced power mode

- Simply controllable

ASIL: B (per ISO 26262-3 Table D.1: S1 + E3 + C1)

FTTI: 200ms

Safety Goal SG-06: The vehicle shall detect battery over-temperature conditions and limit power to prevent progression to dangerous temperatures while maintaining sufficient power for safe vehicle operation.

H-09: Battery Under-Temperature

Hazardous Event: Battery operation at temperatures below safe operating range, reducing performance and potentially causing cell damage

Operational Situation: Cold weather operation (rare per operational data)

Severity (S0):

- Reduced performance (range, power)

- May cause inconvenience but no safety risk in park environment

- No injuries expected

Exposure (E1):

- Very low probability (extreme cold rare in operating environment)

- Once per vehicle lifetime

Controllability (C0):

- Controllable – BMS will disconnect pack

ASIL: QM (per ISO 26262-3 Table D.1: S0)

Safety Goal: None required (QM level)

3.3 High Voltage System Hazards

H-10: HV Isolation Fault - Electric Shock Risk

Hazardous Event: Loss of HV isolation allows current path to chassis, creating electric shock hazard for passengers/driver/maintenance personnel

Operational Situation: Any time HV system is energized, particularly during:

- Normal operation

- Charging

- Maintenance

Severity (S3):

- 500-700V DC can be lethal

- Open-sided vehicle = easy contact with chassis/frame

- Multiple passengers at risk (20 people)

- Fatal injuries likely if shock occurs

Exposure (E1):

- Very low probability with proper isolation monitoring

- LFP chemistry and modern BMS make this rare

- Once per vehicle lifetime probability

Controllability (C3):

- Passengers unaware of invisible electrical hazard

- No warning before contact

- Open-sided design increases contact probability

- Difficult to control or uncontrollable

ASIL: D (per ISO 26262-3 Table D.1: S3 + E1 + C3)

FTTI: 100ms

Safety Goal SG-07: The vehicle shall continuously monitor HV isolation and immediately disable the HV system if isolation falls below safe thresholds.

H-11: HV Contactor Weld/Failure to Open

Hazardous Event: HV contactors fail to open when commanded, leaving HV system energized when it should be off

Operational Situation:

- Attempting to disable HV for safety (after fault detection)

- Shutdown for maintenance

- Emergency situations

Severity (S2):

- Prevents safe shutdown in emergency

- Maintenance personnel at risk

- Could prevent proper response to other faults (e.g., H-05, H-10)

- Severe injuries possible

Exposure (E1):

- Very low probability with proper contactor design

- Once per vehicle lifetime

Controllability (C2):

- System can detect via contactor feedback

- Can prevent operation until resolved

- Maintenance procedures can verify

- Normally controllable with proper procedures

ASIL: C (per ISO 26262-3 Table D.1: S2 + E1 + C2)

FTTI: 100ms

Safety Goal SG-08: The vehicle shall verify HV contactor state and detect contactor weld or failure to open conditions.

3.4 Motor/Drive System Hazards

H-12: Motor/Controller Over-Temperature

Hazardous Event: Motor or motor controller overheats, potentially causing component failure or fire

Operational Situation: Extended high power operation (frequent hill climbing)

Severity (S2):

- Component failure could cause sudden loss of propulsion on hill (see H-02)

- Fire risk if severe overheating (similar consequences to H-05)

- Severe injuries possible

Exposure (E2):

- Low to medium probability given frequent hill climbing

- Proper thermal design should prevent

- Once per year if thermal management inadequate

Controllability (C2):

- Driver can reduce power demand

- System can provide early warnings

- Temperature rises gradually (not instantaneous)

- Normally controllable

ASIL: C (per ISO 26262-3 Table D.1: S2 + E2 + C2)

FTTI: 200ms

Safety Goal SG-09: The vehicle shall monitor motor and motor controller temperatures and limit power or disable drive before dangerous temperatures are reached, while providing adequate warning to the driver.

H-13: Loss of Power Steering Assist

Hazardous Event: Electric power steering system fails, causing sudden increase in steering effort

Operational Situation: Any driving situation, particularly during turning maneuvers or emergency avoidance

Severity (S1):

- Manual steering backup available (hydraulic/mechanical)

- Sudden increase in effort could cause driver to lose control momentarily

- Low speed limits consequences

- Light to moderate injuries possible if driver overcorrects or cannot complete turn

Exposure (E2):

- Low probability with proper power steering design

- Once per year if electrical system has issues

Controllability (C1):

- Professional trained driver can adapt to manual steering

- Low speed allows more time to react

- Manual steering is fully functional, just requires more effort

- Simply controllable for trained operator (>99% can manage)

ASIL: B (per ISO 26262-3 Table D.1: S1 + E2 + C1)

FTTI: 200ms

Safety Goal SG-10: The vehicle shall detect loss of power steering assist and provide driver warning while maintaining manual steering capability.

3.5 Communication System Hazards

H-14: Loss of Communication Between VCU and BMS

Hazardous Event: CAN communication fails between VCU and BMS, preventing critical safety information exchange

Operational Situation: Any driving condition

Severity (S2):

- VCU cannot receive battery alarm levels

- Cannot respond appropriately to battery faults

- Could lead to any battery-related hazard (H-05 through H-09) without proper mitigation

- Severe injuries possible if battery fault not managed

Exposure (E2):

- Low probability with proper CAN bus design

- Single CAN bus (no redundancy) increases risk slightly

- Once per year probability

Controllability (C2):

- VCU can detect timeout and transition to safe state

- Driver may not be aware of communication loss initially

- System behavior depends on fallback strategy

- Normally controllable with proper timeout handling

ASIL: C (per ISO 26262-3 Table D.1: S2 + E2 + C2)

FTTI: 500ms (message timeout threshold)

Safety Goal SG-11: The vehicle shall detect loss of communication between safety-critical components and transition to a safe state that prevents hazardous battery or propulsion conditions.

H-15: Corrupted/Invalid BMS Alarm Level Message

Hazardous Event: VCU receives corrupted or invalid alarm level data from BMS, causing inappropriate response

Operational Situation: Any driving condition

Severity (S2):

- Could cause either:

  - Overly conservative response (unnecessary shutdown) - S0 inconvenience

  - Under-response to real fault (no action when needed) - S2 severe harm

- Worst case is under-response to critical battery fault

Exposure (E2):

- Low probability with CRC/sequence counter protection

- Once per year if CAN bus subject to EMI or software errors

Controllability (C2):

- VCU validation logic can detect implausible values

- Driver may notice inappropriate system behavior

- Normally controllable with proper message validation

ASIL: C (per ISO 26262-3 Table D.1: S2 + E2 + C2)

FTTI: 200ms (validation and response time)

Safety Goal SG-12: The vehicle shall validate all safety-critical messages and detect corrupted or invalid data before taking action.

3.6 Human-Machine Interface Hazards

H-16: Failure to Display Critical Warning

Hazardous Event: HMI fails to display critical warning to driver (alarm level, fault condition)

Operational Situation: When fault condition exists

Severity (S1):

- Driver unaware of developing fault

- Cannot take preventive action

- Could lead to any hazard (H-01 through H-12) but with some delay

- Professional trained driver may notice vehicle behavior changes

- Light to moderate injuries likely (driver has some awareness from training)

Exposure (E2):

- Low probability (display systems generally reliable)

- Once per year if HMI has issues

Controllability (C1):

- Trained driver may notice vehicle behavior (reduced power, unusual sounds/smells)

- Audible alarms provide redundant notification

- Driver can respond to vehicle behavior even without visual warning

- Simply controllable for trained operator

ASIL: B (per ISO 26262-3 Table D.1: S1 + E2 + C1)

FTTI: 200ms (display update time)

Safety Goal SG-13: The vehicle shall provide redundant methods (visual and audible) to notify the driver of critical faults and shall detect HMI communication failures.

3.8 Braking System Interaction Hazards

H-18: Loss of Regenerative Braking

Hazardous Event: Regenerative braking unavailable, increasing wear on friction brakes and potentially catching driver by surprise

Operational Situation: Any braking situation, particularly descending hills (frequent in park)

Severity (S0):

- Friction brakes (service + parking) remain fully functional

- Redundant braking system adequate for vehicle weight/speed

- No injuries expected

- Inconvenience only (increased brake wear)

Exposure (E3):

- Medium probability if drive system has issues

- Could occur monthly

Controllability (C0):

- Driver notices different pedal feel

- Friction brakes adequate

- Controllable in general

ASIL: QM (per ISO 26262-3 Table D.1: S0)

Safety Goal: None required (QM level)

Note: Should provide driver notification of regen brake unavailability.

3.9 Charging System Hazards

H-19: Uncontrolled Charging (Over-Current/Over-Voltage)

Hazardous Event: Charging system provides excessive current or voltage to battery, causing cell damage or thermal event

Operational Situation: During charging (typically with no passengers aboard)

Severity (S1):

- Occurs during charging when vehicle unoccupied

- May damage battery or cause delayed thermal event

- Maintenance/charging personnel at risk

- Light to moderate injuries possible

Exposure (E2):

- Low probability with proper BMS and charger communication

- Once per year if charging control inadequate

Controllability (C1):

- BMS monitors and can terminate charging

- Charging occurs in controlled environment

- Personnel can disconnect charger

- Simply controllable

ASIL: B (per ISO 26262-3 Table D.1: S1 + E2 + C1)

FTTI: 100ms (charging control response)

Safety Goal SG-15: The vehicle shall monitor and limit charging current and voltage to prevent battery damage and unsafe conditions.

4. Summary of Safety Goals

| ID | ASIL | FTTI | Safety Goal |
| --- | --- | --- | --- |
| SG-01 | C | 100ms | The vehicle shall prevent unintended acceleration and ensure driver commands have priority over all other control inputs. |
| SG-02 | B | 200ms | The vehicle shall provide sufficient warning before loss of propulsion to allow driver to safely stop, and shall not disable propulsion without adequate notice except in immediate danger situations. |
| SG-03 | C | 100ms | The vehicle shall prevent unintended braking and ensure brake application only occurs in response to driver command or emergency automatic braking in imminent collision scenarios. |
| SG-04 | D | 50ms | The vehicle shall detect battery over-temperature conditions and transition to a safe state (disable charging/discharging) before thermal runaway occurs. |
| SG-05 | C | 100ms | The vehicle shall prevent battery cell voltages from exceeding safe limits during all charging and discharging operations. |
| SG-06 | B | 200ms | The vehicle shall detect battery over-temperature conditions and limit power to prevent progression to dangerous temperatures while maintaining sufficient power for safe vehicle operation. |
| SG-07 | D | 100ms | The vehicle shall continuously monitor HV isolation and immediately disable the HV system if isolation falls below safe thresholds. |
| SG-08 | C | 100ms | The vehicle shall verify HV contactor state and detect contactor weld or failure to open conditions. |
| SG-09 | C | 200ms | The vehicle shall monitor motor and motor controller temperatures and limit power or disable drive before dangerous temperatures are reached, while providing adequate warning to the driver. |
| SG-10 | B | 200ms | The vehicle shall detect loss of power steering assist and provide driver warning while maintaining manual steering capability. |
| SG-11 | C | 500ms | The vehicle shall detect loss of communication between safety-critical components and transition to a safe state that prevents hazardous battery or propulsion conditions. |
| SG-12 | C | 200ms | The vehicle shall validate all safety-critical messages and detect corrupted or invalid data before taking action. |
| SG-13 | B | 200ms | The vehicle shall provide redundant methods (visual and audible) to notify the driver of critical faults and shall detect HMI communication failures. |
| SG-14 | C | N/A | The vehicle shall monitor fire detection system integrity and detect failures through periodic self-test. Fire-wire circuit shall use fault-detection methods to distinguish between inactive, active, and fault conditions. |
| SG-15 | B | 100ms | The vehicle shall monitor and limit charging current and voltage to prevent battery damage and unsafe conditions. |

5. ASIL Distribution

ASIL D (Highest): 2 hazards

- H-05: Battery thermal runaway / fire

- H-10: HV isolation fault

ASIL C: 8 hazards

- H-01: Unintended acceleration

- H-04: Unintended braking

- H-06: Battery over-voltage

- H-11: HV contactor failure

- H-12: Motor/controller over-temperature

- H-14: VCU-BMS communication loss

- H-15: Corrupted BMS messages

ASIL B: 6 hazards

- H-02: Loss of propulsion on hill

- H-08: Battery over-temperature (non-fire)

- H-13: Loss of power steering assist

- H-16: Failure to display warnings

- H-19: Uncontrolled charging

QM (No ASIL): 4 hazards

- H-03: Loss of propulsion on flat

- H-07: Battery under-voltage

- H-09: Battery under-temperature

- H-18: Loss of regenerative braking

6. Key Risk Mitigating Factors

Your vehicle has several favorable characteristics that reduce risk:

- Low Speed (25 mph): Significantly reduces kinetic energy and severity of collisions

- Controlled Environment: Park setting eliminates many public road hazards

- Professional Trained Drivers: High controllability for many hazards

- Open-Sided Design: Excellent emergency egress capability

- Low Annual Mileage: Reduces exposure probability

- Redundant Braking: Independent backup for loss of propulsion

- Manual Steering Backup: Maintains control even if power steering fails

- LFP Chemistry: More thermally stable than other lithium-ion chemistries

7. Assumptions and Limitations

This HARA assumes:

- Drivers receive proper training on EV-specific behaviors and emergency procedures

- Regular maintenance is performed per schedule

- Battery BMS functions correctly (separate safety analysis required for BMS)

- Vehicle operates only in controlled park environment (not public roads)

- Maximum speed governed to 25 mph

- Charging occurs in designated areas with trained personnel

If any of these assumptions change, this HARA must be reviewed and updated.

8. Next Steps - Functional Safety Concept

The Safety Goals identified above must now be allocated to functional safety requirements in the Functional Safety Concept document. This will define:

- High-level safety mechanisms for each safety goal

- Safe states the vehicle can transition to

- Fault detection and response strategies

- FTTI (Fault Tolerant Time Interval) for each hazard (defined in Section 4)

- Allocation to systems (VCU, BMS, motor controller, etc.)

After the Functional Safety Concept, Technical Safety Requirements (like your VCU requirements document) will specify the detailed implementation.

Document Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Safety Manager |  |  |  |
| Chief Engineer |  |  |  |
| Quality Manager |  |  |  |

Document Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2025-10-29 | [Author] | Initial HARA for park shuttle electric retrofit |
| 2.0 | 2025-10-30 | [Author] | Removed generic hazards H-18 through H-22; Added H-04 (Unintended Braking), H-13 (Loss of Power Steering), Added FTTI values; Added ISO 26262-3 Table D.1 risk matrix; Updated safety goal numbering |